VERIFICATION PLAN

Parameterized Up/Down Counter

SystemVerilog RTL | UVM 1.2 | SVA | Functional Coverage

| DUT | Parameterized synchronous up/down counter |
| --- | --- |
| Default parameter | WIDTH = 4 for verification |
| Verification methodology | UVM 1.2 with scoreboard, SVA, and functional coverage |
| Final result | 549 checks passed, 0 failed |
| Coverage result | 100.00% functional coverage |

Project status: Verification complete; documentation and repository packaging remain.

# 1. Purpose

This verification plan defines the strategy, architecture, test scenarios, assertions, functional coverage, checking methodology, and sign-off criteria for the parameterized up/down counter RTL. The goal is to verify correct behavior for reset, parallel load, count-up, count-down, hold, and natural fixed-width rollover.

# 2. DUT Overview

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | Input | 1 | Positive-edge clock |
| rst_n | Input | 1 | Asynchronous active-low reset |
| enable | Input | 1 | Enables counting when load is inactive |
| load | Input | 1 | Loads load_data; has priority over enable |
| up_down | Input | 1 | 1: count up, 0: count down |
| load_data | Input | WIDTH | Parallel value loaded into count |
| count | Output | WIDTH | Current counter value |

Control priority: reset > load > enable > hold.

Rollover behavior: The counter uses natural fixed-width arithmetic. Incrementing the maximum value wraps to zero; decrementing zero wraps to the maximum value.

# 3. Verification Objectives

Verify asynchronous active-low reset forces count to zero.

Verify parallel load updates count with load_data.

Verify load has priority when load and enable are asserted together.

Verify count increments by one when enable=1 and up_down=1.

Verify count decrements by one when enable=1 and up_down=0.

Verify count holds when load=0 and enable=0.

Verify up-count rollover from maximum to zero.

Verify down-count rollover from zero to maximum.

Verify count never contains X or Z after reset is released.

Measure functional coverage for operations, boundaries, directions, transitions, and priority behavior.

# 4. Verification Methodology

The environment uses UVM 1.2. Stimulus is generated by directed and constrained-random sequences. The driver applies transactions through a clocking block, the monitor samples DUT activity, the scoreboard maintains a reference model, the coverage subscriber samples functional coverage, and SystemVerilog assertions independently check temporal behavior.

| Component | Responsibility |
| --- | --- |
| counter_seq_item | Carries reset, enable, load, direction, load data, and observed count. |
| counter_sequencer | Arbitrates and supplies sequence items to the driver. |
| counter_driver | Drives DUT input signals at the clocking-block event. |
| counter_monitor | Samples DUT inputs and output and broadcasts observed transactions. |
| counter_agent | Contains active driver/sequencer and passive monitor. |
| counter_scoreboard | Predicts expected count and compares it against DUT count. |
| counter_coverage | Collects functional coverage from monitored transactions. |
| counter_env | Connects agent, scoreboard, and coverage subscriber. |
| counter_assertions | Checks reset, load, count, hold, rollover, and known-value properties. |
| counter_full_test | Runs all major verification scenarios sequentially in one simulation. |

# 5. Test Strategy

The final regression is executed through counter_full_test. Each major scenario is started once, one after another, in a single UVM run.

| ID | Scenario | Stimulus | Expected behavior | Status |
| --- | --- | --- | --- | --- |
| TC-01 | Reset | Assert rst_n low for multiple cycles, then release | count remains zero during reset | PASS |
| TC-02 | Load | load=1, load_data=0x5 | count loads 0x5 | PASS |
| TC-03 | Count up | enable=1, up_down=1 across full range | count increments and wraps max→0 | PASS |
| TC-04 | Count down | enable=1, up_down=0 across full range | count decrements and wraps 0→max | PASS |
| TC-05 | Hold | enable=0, load=0 | count remains unchanged | PASS |
| TC-06 | Random | 500 constrained-random transactions | all legal combinations follow priority and arithmetic rules | PASS |

# 6. Scoreboard and Reference Model

The scoreboard maintains expected_count as a WIDTH-bit reference model. For each monitored transaction, it compares the observed count against the predicted value and then updates the reference model according to the DUT priority.

If rst_n=0, expected_count becomes zero.

Else if load=1, expected_count becomes load_data.

Else if enable=1 and up_down=1, expected_count increments by one.

Else if enable=1 and up_down=0, expected_count decrements by one.

Else expected_count holds its previous value.

Final scoreboard result: 549 comparisons passed and 0 comparisons failed.

# 7. Assertion Plan

| Assertion | Intent | Expected result |
| --- | --- | --- |
| p_reset | Reset forces count to zero | Pass |
| p_load | Load transfers previous-cycle load_data into count | Pass |
| p_count_up | Enabled up-count increments by one | Pass |
| p_count_down | Enabled down-count decrements by one | Pass |
| p_hold | Disabled counter holds its value | Pass |
| p_up_rollover | Maximum up-count wraps to zero | Pass |
| p_down_rollover | Zero down-count wraps to maximum | Pass |
| p_count_known | count contains no X/Z after reset release | Pass |

# 8. Functional Coverage Plan

| Coverage item | Bins / conditions | Purpose |
| --- | --- | --- |
| reset_cp | asserted, deasserted | Confirm reset is exercised |
| enable_cp | disabled, enabled | Cover count enable states |
| load_cp | inactive, active | Cover parallel load states |
| direction_cp | up, down, up→down, down→up | Cover direction and transitions |
| operation_cp | reset, load, hold, count up, count down | Cover all operations |
| load_data_cp | zero, maximum, other values | Cover important load values |
| count_cp | zero, maximum, other values | Cover counter boundaries and normal values |
| up_rollover_stimulus_cp | count=max during up operation | Confirm up rollover stimulus |
| down_rollover_stimulus_cp | count=0 during down operation | Confirm down rollover stimulus |
| load_enable_cross | load active × enable enabled | Verify load-priority combination |

Final functional coverage: 100.00%.

# 9. Pass/Fail and Sign-Off Criteria

| Category | Criterion | Result |
| --- | --- | --- |
| Compilation | No compile errors | Met |
| Simulation | Test completes normally | Met |
| Scoreboard | 0 mismatches | Met |
| Assertions | No assertion errors | Met |
| Functional coverage | 100% for the defined covergroup | Met |
| UVM report | 0 UVM_ERROR and 0 UVM_FATAL | Met |

# 10. Final Verification Results

| Test executed | counter_full_test |
| --- | --- |
| Functional scenarios | Reset, load, count up, count down, hold, random |
| Scoreboard | 549 passed, 0 failed |
| Functional coverage | 100.00% |
| UVM errors/fatals | 0 / 0 |
| Verification status | SIGNED OFF |

# 11. Remaining Project-Completion Items

The RTL and verification are complete. The following items are recommended to make the repository presentation-ready and interview-ready:

Add this verification plan under docs/verification_plan.md and/or docs/verification_plan.docx.

Create a concise README with project overview, architecture, folder structure, run instructions, tests, and final results.

Save the successful simulation log under results/counter_full_test.log.

Add one clean waveform screenshot showing reset, load, up-count, down-count, hold, and rollover.

Run a lint check and document any warnings or waivers.

Optionally collect code coverage separately; the current 100% result is functional coverage, not RTL code coverage.

Push the final source, documentation, and results to GitHub with a clear commit message.

# 12. Recommended Repository Documents

| File | Contents |
| --- | --- |
| README.md | Overview, features, architecture, run command, results, repository structure |
| docs/design_specification.md | DUT interface, priority, arithmetic, reset, rollover behavior |
| docs/verification_plan.md | Verification objectives, tests, assertions, coverage, sign-off |
| docs/testplan.md | Testcase matrix with stimulus, checks, expected result, final status |
| results/counter_full_test.log | Successful UVM regression output |
| docs/waveforms/ | Selected waveform screenshots and explanations |